Orifice information

The following material was taken directly from my burner and Forge notebook.    Calculating the precise amount of air or gas flowing through an orifice is fairly complicated.   I would like to present the following information as notes, I have taken over the years.  I have not found any detailed information directly relation to burner orifice design.  I have found bits and pieces regarding orifice shapes, which I would like to share with you.

Flow through a thin plate orifice

Thin plate orifice: 		Fig. 1

- Most common and easy to make

- Thin plate orifice usually .040 to .0625 thick

- Should be machined

- No burs or sharp edges

- Orifice must be a clean hole  (hand ream)?

- Thicker plates, .080 may drop the coefficient to .50

- Thicker than .080 could be as low as .45

Re-entrant orifice:		Fig. 2

- Inside tube length 20% to 50% of diameter

- Tube should be precision placed 90 degrees to orifice plate

- No sharp edges, no burs

- Tube should be square on both sides

- Tube should be flush and square with outside plate  (almost a press fit)

Square edge:		Figure 3

- See figure 3 comments

Round edge		Fig. 4

- Common in the industry and favored for fluid and gas

- Length of tube 20% to 50% of diameter

- Note round edge improves laminar flow

- May be possible to build a punch and or die to create smooth contour

- Polish input and inside orifice

- Orifice output should be square to plate

Extended round edge		Fig. 5

- Similar to fig. 4

- Length of extended orifice tube 50% to 75% of diameter

Inside taper		Fig. 6

- Offers some improvement over square edge

- Need to determine the entry angle for best performance

- Plate thickness should be thin.

Inside taper		Fig. 7

- No real data, needs additional research

- Offers some improvement over square edge

- Need to determine the entry angle for best performance

- Plate thickness should be thin.

I Hope this information may be helpful for some of you.

Thomas Vincent

San Antonio, TX.